Cessna 172M N515ED     Checklists 					    Cessna 172M N515ED     Checklists

Preflight

Control Wheel Lock           removed

Ignition switch                             off

Master switch		                 on

Fuel gauges			check

Lights				check

Stall horn			check

Pitot heat			check

Master switch		                 off

AROW Documents 	            check

Left wing fuel sump		  drain

Elevator hinge bolts		check

Rudder hinge bolts		check

Trim pushrod & bolts		check

Right wing fuel sump		  drain

Right wheel, tire, brakes	check

Right flap hinge & pushrod	check

Right aileron & hinges		check

Right fuel tank & quantity	check

Oil level		        6 - 8 qts

Main fuel sump		 drain

Propeller and spinner	        checked

Air filter/cowling	unobstructed

Nose wheel & strut		check

Static vent	            unobstructed

Left fuel tank & quantity	check

Pitot tube                     unobstructed

Fuel Overflow              unobstructed

Left aileron & hinges		check

Left flap hinge & pushrod	check

Left wheel, tire, brakes	check

Baggage door		           locked

Before Starting Engine

Passenger briefing	       complete

Seatbelts, Comfort (vents, heat), Fire Extinguisher, Emergency Exit

Seats & belts	        adjust & secure

Doors & Windows	          latched

Parking brake			     set

Avionics master                           off

Fuel selector			   both

Elevator trim                                set

Starting Engine

Flaps                                 retracted

Carb heat			  cold

Throttle	             cracked 1/8”

Mixture			   rich

Prime	       as needed (2-4 strokes)

Propeller area			  clear

Master switch			     on

Rotating beacon		     on

Ignition switch			  start

Don’t crank for more than 10 seconds!

Oil pressure   positive within 30 sec

Avionics master switch                on

If Engine Over-Primed/Flooded

Mixture                               Full lean

Throttle                              Full open

Starter          Crank through several revolutions

Repeat starting procedure

Engine Fire During Start

Ignition 	     continue cranking

Throttle			     full

Mixture		      idle cutoff

Master switch			      off

Fuel selector			      off

Ignition				      off

Use fire extinguisher       as needed

Taxi

Taxi & clearance lights	     on

Parking brake		       released

Brakes		                           test

Control position    for wind direction

Backup attitude    normal operation

Before Take Off Run Up

Parking brake			     set

Controls		free & correct

Fuel selector			   both

Mixture			    rich

Before Take Off Run Up, cont

Throttle	   	      1700 rpm

Engine instruments                check

Magnetos			check

(max drop<125, difference <50 rpm)

Carb heat			check

Vacuum		         5.0” Hg

Throttle                            idle check

Flaps	                    check retracted

Elevator trim             set for take-off

Cabin doors	       closed & latched

Seat belts & harnesses         secure

Radios	 & Navigation    	     set

Navigation source                confirm

Backup instruments      set & check

Transponder                    set to ALT

Parking brake		       released

Take Off & Climb

Landing light			     on

Flaps*    	                             0°

Throttle	     	      full power

Take off roll – airspeed alive & engine instruments green

Rotate			         60 mph Climb                              75-85 mph

Max Performance Take Off

Flaps*                                           0°

Brakes                                      held

Power                             full throttle

Brakes                               released

Climb (clear of obstacles)   68 mph

* Normal and obstacle clearance takeoffs are performed with wing flaps up.  The use of 10° flaps is reserved for minimum ground runs or for takeoffs from soft or rough fields (ref. POH page 2-14)

Engine Failure After Takeoff

PUSH!!         forward yoke pressure

Airspeed (no flaps)	         75 mph

Mixture		     idle cut off

Fuel selector		 	     off

Master switch			     off

Flaps                  40° recommended

Ignition switch			     off

Don’t turn back!!

Cruise

Power	                  2200 - 2700 rpm

(no more than 75% power)

Mixture	            lean carefully

Trim				adjust

Engine Failure During Flight

Fly the airplane!!!

Airspeed		         80 mph

Carb heat			     on

Fuel selector			  both

Mixture			   rich

Ignition switch			  both

Primer			   in & locked

Cabin doors           propped open

Prepare for off-field landing

Engine Fire in Flight

Fly the airplane; maneuver to land immediately!!!

Mixture		    idle cutoff

Fuel selector			     off

Master switch                            off

Cabin heat/air	                off

Airspeed		       120 mph

Cabin doors           propped open

Prepare for off-field landing

Cessna 172M N515ED     Checklists

Electrical Fire in Flight

Fly the airplane!!!

Master switch	                off

All electrical equip	                off

Avionics                      	     off

Vents/Cabin heat          closed/off

Use fire extinguisher         if needed

Ventilate cabin

Circuit breakers                  checked

Land as soon as possible

Descent

Mixture		           enrich

Power			    as needed

Carb heat		    as needed

Before Landing

Fuel selector			  both

Mixture			   rich

Carb heat			     on

Landing lights			     on

Seat belts		          secure

Airspeed (flaps up)      70 – 80 mph

Wing flaps                      as desired

Airspeed (flaps down)  65 – 75 mph

Balked Landing

Throttle                                        full

Carb heat                                  cold

Wing flaps                                  20°

Airpspeed                            65 mph

Wing flaps                  retract slowly

After Landing

(clear of the runway)

Flaps			           retract

Carb heat			   cold

Transponder                       standby

Trim                          set for take off

Landing light		  	     off

Taxi light	               as required

Shut Down & Securing Aircraft

Avionics master switch	     off

Lights		                        all off Throttle		      1000 rpm

Mixture		    idle cut-off

Ignition switch			     off

Master	switch			     off

Property of:

Valley Flyers

Just Plane Fun

If Found, please call 503-559-4593, or

Email: info@valleyflyers.org

Mailing Address: 885 Lancaster Drive SE,

Suite D

Salem, OR 97317

Brakes				    set

Aircraft refueled, cleaned and hangared

Control lock and tie downs when parking outside

Important Information

Vr                                         60 mph

Vx                                        68 mph

Vy (at sea level)                   91 mph

Va (at gross wt)                 112 mph

Vfo                                     100 mph

Best Glide                            80 mph

Gross weight                         2300#

Utility Category                      2000#

Empty weight                         1478#

Useful load                              822#

CG                                          40.12

Fuel Capacity (usable)           38 gal

Tire Pressures              Mains 29psi

Nose 31 psi

Phone numbers:

Joan Johnson	          503-559-4593

Al Gray                      503-932-9374

Salem Tower            503-581-3571

Salem Air Fuel          503-364-4158

Flight Service            800-992-7433

Portland FSDO         503-615-3200