Piper PA28-140 N1636H      Checklists 				 Piper PA28-140 N1636H      Checklists

Preflight

Master switch		     on

Fuel gauges			check

Lights				check

Stall horn			check

Pitot heat			check

Master switch		     off

AROW Documents 	            check

Right flap               	            check

Right aileron    		check

Fuel overflow        check unplugged

Right wing fuel sump		  drain

Right fuel tank & quantity	check

Right wheel, tire, brakes	check

Oil level		        6 - 8 qts

Propeller and spinner	        checked

Air filter/cowling	unobstructed

Nose wheel & strut		check

Main fuel sump		 drain

Left wing fuel sump		  drain

Left fuel tank & quantity	check

Pitot/static tube           unobstructed

Left aileron  		            check

Left flap  	                        check

Left wheel, tire, brakes	check

Stabilator         		check

Antiservo tab hinges               check

Rudder hinge bolts		check

Before Starting Engine

Preflight inspection	       complete

Passenger briefing	       complete

Seatbelts, Comfort (vents, heat), Fire Extinguisher, Emergency Exit

Seats & belts	        adjust & secure

Door & Window	          latched

Parking brake			     set

Avionics                                       off

Fuel selector		    fullest tank

Elevator trim              set for take off

Starting Engine

Carb heat			  cold

Mixture			   rich

Throttle	             cracked 1/4”

Prime	       as needed (2-4 strokes)

Propeller area			 clear

Master switch			     on

Electric fuel pump                        on

Rotating beacon		     on

Ignition switch		        engage*

Oil pressure   positive within 30 sec

Electric fuel pump                        off

Avionics                                       on *Don’t crank for more than 10 seconds!

If Engine Over-Primed/Flooded

Throttle                              Full open

Mixture                               Full lean

Electric fuel pump                       off

Starter                                 engage

Mixture                               advance

Throttle                                  retard

Engine Fire During Start

Ignition 	     continue cranking

Throttle			     full

Mixture		                   idle cutoff

Master switch			      off

Electric fuel pump                        off

Fuel selector			      off

Ignition				      off

Use fire extinguisher       as needed

Taxi

Taxi & clearance lights	     on

Parking brake		       released

Brakes		                           test

Control position    for wind direction

Heading indicator  normal operation

Turn coordinator   normal operation

Before Take Off Run Up

Parking brake			     set

Controls		free & correct

Fuel selector	                fullest tank

Mixture			    rich

Throttle	   	      2000 rpm

Engine instruments                check

Magnetos			check

(max drop<175, difference <50 rpm)

Carb heat			check

Vacuum		          5.0” hg

Annunciator panel	  press to test

Throttle                            idle check

Elevator trim             set for take-off

Cabin door	       closed & latched

Seat belts		          secure

Instruments		     rechecked

Transponder                    set to ALT

Parking brake		       released

Take Off & Climb

Landing lights			     on

Electric fuel pump                        on

Flaps 	   	                              0°

Throttle	     	      full power

Take off roll – airspeed alive & engine instruments green

Rotate			   45-55 KIAS Climb                          Vx = 66 KIAS

Vy = 75 KIAS

Electric fuel pump	 off at altitude

Max Performance Take Off

Flaps                                          25°

Brakes                                      held

Power                             full throttle

Brakes                               released

Climb (clear of obstacles)   66 KIAS

Flaps           retract above 200’ AGL

Electric fuel pump       off at altitude

Engine Failure After Takeoff

PUSH!!         forward yoke pressure

Airspeed (no flaps)	         75 mph

Mixture		     idle cut off

Fuel selector		 	     off

Master switch			     off

Flaps                  40° recommended

Ignition switch			     off

Don’t turn back!!

Cruise

Power	                  2000 - 2600 rpm

(no more than 75% power)

Mixture	            lean carefully

Trim				adjust

Landing lights			     off

Switch Fuel Tanks

Check about every ½ hour

Electric fuel pump                       on

Switch tanks

Electric fuel pump                       off

Engine Failure During Flight

Fly the airplane!!!

Electric fuel pump                        on

Fuel selector                switch tanks

Mixture			   rich

Carb heat			     on

Ignition switch			  both

Primer			   in & locked

Airspeed		        69 KIAS

Cabin door             propped open

Prepare for off-field landing

Engine Fire in Flight

Fly the airplane!!!

Airspeed		      100 KIAS

Mixture		    idle cutoff

Master switch                            off

Electric fuel pump                    off

Fuel selector			    off

Vents & windows                   closed Cabin heat/defroster    	     off

Cabin door             propped open

Land immediately

Descent

Mixture		           enrich

Power			    as needed

Carb heat		    as needed

Before Landing

Fuel selector                   fullest tank

Electric fuel pump                        on

Mixture			    rich

Carb heat			 apply

Landing lights			     on

Seat belts		          secure

Airspeed (flaps up)             70 KIAS

Airspeed ( “ extended)        59 KIAS

Trim				adjust

After Landing

(clear of the runway)

Flaps			           retract

Carb heat			   cold

Electric fuel pump                        off

Transponder                        standby

Trim                           set for take off

Landing light		  	     off

Taxi light	               as required

Shut Down & Securing Aircraft

Avionics                      	     off

Throttle		     1000 rpm

Mixture		    idle cut-off

Ignition switch			     off

Lights				 all off

Property of:

Valley Flyers

Just Plane Fun

If Found, please call 503-559-4593, or

Email: info@valleyflyers.org

Mailing Address: 885 Lancaster Drive SE,

Suite D

Salem, OR 97317

Master	switch			     off

Brakes				    set

Important Information

Vr                                   45-55 KIAS

Vx                                       66 KIAS

Vy (at sea level)                 75 KIAS

Va (at gross wt)                114 KIAS

Vfe                                    101 KIAS

Best Glide                           69 KIAS

Gross weight                         2150#

Utility Category                      1950#

Empty weight                         1437#

Useful load                              713#

Empty Arm                               85.2”

Fuel Capacity (usable)        49.6 gal

Fuel Capacity (at the tabs)     36 gal

Important Phone numbers:

Joan Johnson	    503-559-4593

Al Gray                503-932-9374

Salem Tower       503-581-3571

Salem Air Fuel    503-364-4158

Flight Service      800-992-7433

Portland FSDO   503-615-3200